Perfect—that board is a great match. The Seeed XIAO ESP32-S3 runs the Arduino ESP32 core and works very well with the NimBLE-Arduino library, so the NimBLE rewrite I sent is the right direction. I’ve tailored it below for the XIAO-ESP32-S3 with a couple of ESP32-specific cleanups and comments. The NeoPixel logic and your area/color mapping remain unchanged from your original sketch. [utsystemad...epoint.com]

Notes specific to XIAO-ESP32-S3

- Pins: Your file used pins {7, 9} for the strips. On ESP32-S3 those refer to GPIO7 and GPIO9 (they exist on S3), but double-check they’re actually routed on your XIAO board header and not used by USB/JTAG on your variant. If your LEDs are on different pins (e.g., D1/D2), just update the pins[] array. [utsystemad...epoint.com]

- Random seed: analogRead(0) isn’t meaningful on ESP32-S3 like it was on some other boards. I switched to esp_random() for a proper hardware RNG seed. [utsystemad...epoint.com]

- 5V data line: The S3 is 3.3V; most WS2812B strips work fine with a 3.3V data line if power and wiring are short, but a level shifter is recommended for long runs or if you see flicker. (No code changes needed—just a wiring note.) [utsystemad...epoint.com]

XIAO-ESP32-S3 NimBLE version

#include <Arduino.h>

#include <NimBLEDevice.h>

#include <Adafruit_NeoPixel.h>

#include "esp_random.h"   // for random seed on ESP32

// ----------------------- NeoPixel configuration -----------------------

#define STRIPCOUNT 2

#define PIXELCOUNT 3

#define NEOBRIGHT 75

#define PIXELFORMAT NEO_GRB + NEO_KHZ800

Adafruit_NeoPixel strip[STRIPCOUNT];

// Update these to whatever GPIOs your two strips are actually on

// On XIAO-ESP32-S3, verify these GPIOs are available on your header and free.

uint8_t pins[STRIPCOUNT] = {7, 9};  // GPIO7, GPIO9

// ----------------------- Device Info -----------------------

const char* DEVNAME = "KyberVault";

// ----------------------- Timers & Counters -----------------------

#define CHANGEDELY 51000     // 5 second Delay (was 15s in original)  1

// ----------------------- Location IDs -----------------------

#define NOBEACON     0

#define MARKETPLACE  1

#define DROIDDEPOT   2

#define RESISTANCE   3

#define UNKNOWN      4

#define ALERT        5

#define DOKONDARS    6

#define FIRSTORDER   7

// ----------------------- Crystal Colors -----------------------

#define BLACK       0x00000000

#define WHITE       0x00FFFFFF

#define RED         0x00FF0000

#define CRIMSONRED  0x00DC143C

#define PURPLE      0x00FF00FF

#define DARKPURPLE  0x00301934

#define BLUE        0x0000008B

#define CYAN        0x0000FFFF

#define GREEN       0x0000FF00

#define YELLOW      0x00FFBF00

#define ORANGE      0x00FF7E00

// ----------------------- Filters -----------------------

#define RSSI           -75         // Minimum RSSI to consider   1

#define BLE_DISNEY     0x0183      // Manufacturer Company ID    1

const String IGNOREHOST = "SITH-TLBX"; // Ignore a specific beacon host  1

// ----------------------- State -----------------------

uint32_t last_activity;

int8_t   scan_rssi;

uint16_t area_num = 0, last_area_num = 9;

String   beacon_name = "";

// ----------------------- Forward Declarations -----------------------

void updatePixels();

void colorPulse();

// ----------------------- NimBLE scanning callback -----------------------

class MyAdvertisedDeviceCallbacks : public NimBLEAdvertisedDeviceCallbacks {

void onResult(NimBLEAdvertisedDevice dev) override {

// RSSI filter

int rssi = dev->getRSSI();

if (rssi < RSSI) return;

// Manufacturer data: first 2 bytes = company ID (little-endian on BLE advertising)

std::string mfg = dev->getManufacturerData();

if (mfg.size() < 5) return; // we need at least [0]=LSB company, [1]=MSB, [2]=type, [3..]=payload

const uint8_t* md = reinterpret_cast<const uint8_t*>(mfg.data());

uint16_t company = (uint16_t)md[1] << 8 | md[0];

if (company != BLE_DISNEY) return;

// Your original beacon type check: md[2] == 0x0A for "Location"

if (md[2] != 0x0A) return;

// Name (Complete Local Name) — comes via scan response if active scan is enabled

std::string n = dev->getName();

beacon_name = n.c_str();

// Host ignore & timing guard

if (beacon_name == IGNOREHOST) return;

if ((millis() - last_activity) < CHANGEDELY) return;

last_activity = millis();

// Area/location ID is at md[4] in your original payload layout

area_num = md[4];

scan_rssi = rssi;

// // Debug (optional)

// Serial.printf("Host: %s  RSSI:%d  Area:%u\n", beacon_name.c_str(), scan_rssi, area_num);

}

};

static MyAdvertisedDeviceCallbacks advCB;

void setup() {

Serial.begin(115200);

// while (!Serial) delay(10);

Serial.println("SWGE_Beacon_Scan (NimBLE on XIAO-ESP32-S3)");

Serial.println("----------------\n");

last_activity = CHANGEDELY;

// ----------------------- NeoPixel init -----------------------

for (int x = 0; x <= 1; x++) {

strip[x].updateLength(PIXELCOUNT);

strip[x].updateType(PIXELFORMAT);

strip[x].setPin(pins[x]);

}

for (int x = 0; x <= 1; x++) {

strip[x].begin();

strip[x].show();

for (int p = 0; p <= 2; p++) {

strip[x].setPixelColor(p, BLACK);

}

strip[x].setBrightness(25);

strip[x].show();

}

// Better seed for ESP32

randomSeed((uint32_t)esp_random());

// ----------------------- NimBLE init -----------------------

NimBLEDevice::init(DEVNAME);          // sets the local name, too  1

NimBLEDevice::setPower(ESP_PWR_LVL_P3); // optional: TX power level; adjust if needed

NimBLEScan* scan = NimBLEDevice::getScan();

scan->setAdvertisedDeviceCallbacks(&advCB, false); // false = skip duplicates

scan->setActiveScan(true);     // needed to get scan responses (for names)  1

scan->setInterval(160);        // 1600.625ms=100ms, matching your original  1

scan->setWindow(80);           // 800.625ms = 50ms                         1

scan->setDuplicateFilter(true);

// Start scanning continuously (no auto-stop)

scan->start(0, nullptr, false);

Serial.println("Starting SWGE Location Scanning ...");

}

void updatePixels() {

switch (area_num) {

case MARKETPLACE:

strip[0].setPixelColor(0, GREEN);

strip[1].setPixelColor(0, BLUE);

strip[0].setPixelColor(1, YELLOW);

strip[1].setPixelColor(1, DARKPURPLE);

strip[0].setPixelColor(2, CYAN);

strip[1].setPixelColor(2, ORANGE);

break;

case ALERT:

strip[0].setPixelColor(0, YELLOW);

strip[1].setPixelColor(0, ORANGE);

strip[0].setPixelColor(1, CYAN);

strip[1].setPixelColor(1, YELLOW);

strip[0].setPixelColor(2, ORANGE);

strip[1].setPixelColor(2, CYAN);

break;

case RESISTANCE:

strip[0].setPixelColor(0, BLUE);

strip[1].setPixelColor(0, GREEN);

strip[0].setPixelColor(1, CYAN);

strip[1].setPixelColor(1, PURPLE);

strip[0].setPixelColor(2, WHITE);

strip[1].setPixelColor(2, YELLOW);

break;

case UNKNOWN:

for (uint16_t s = 0; s <= STRIPCOUNT - 1; s++) {

for (uint16_t p = 0; p <= PIXELCOUNT - 1; p++) {

strip[s].setPixelColor(p, WHITE);

}

}

break;

case DROIDDEPOT:

strip[0].setPixelColor(0, CYAN);

strip[0].setPixelColor(1, BLUE);

strip[0].setPixelColor(2, CYAN);

for (uint16_t p = 0; p <= PIXELCOUNT - 1; p++) {

strip[1].setPixelColor(p, GREEN);

}

break;

case DOKONDARS:

strip[0].setPixelColor(0, PURPLE);

strip[1].setPixelColor(0, BLUE);

strip[0].setPixelColor(1, GREEN);

strip[1].setPixelColor(1, DARKPURPLE);

strip[0].setPixelColor(2, BLUE);

strip[1].setPixelColor(2, GREEN);

break;

case FIRSTORDER:

for (uint16_t s = 0; s <= STRIPCOUNT - 1; s++) {

for (uint16_t p = 0; p <= PIXELCOUNT - 1; p++) {

strip[s].setPixelColor(p, RED);

}

}

break;

case NOBEACON:

default:

strip[0].setPixelColor(0, PURPLE);

strip[1].setPixelColor(0, BLUE);

strip[0].setPixelColor(1, CYAN);

strip[1].setPixelColor(1, GREEN);

strip[0].setPixelColor(2, YELLOW);

strip[1].setPixelColor(2, ORANGE);

break;

}

for (uint16_t s = 0; s <= STRIPCOUNT - 1; s++) {

strip[s].setBrightness(NEOBRIGHT);

strip[s].show();

}

}

void colorPulse() {

int stripNumber = random(0, 2);

int iDelay = random(50, 201);

int iBright = random(150, 256);

delay(iDelay);

strip[stripNumber].setBrightness(iBright);

strip[stripNumber].show();

}

void loop() {

if (area_num != last_area_num) {

updatePixels();

last_area_num = area_num;

}

colorPulse();

}

Build checklist (Arduino IDE / PlatformIO)

- Board: Seeed XIAO ESP32S3 (Arduino ESP32 core).

- Library: Install NimBLE-Arduino (by h2zero) via Library Manager.

- USB CDC On Boot: If serial prints are missing, try toggling this in Tools → USB CDC On Boot for your core version.

- Power: Provide stable 5V to the LEDs; connect GND between XIAO and the strips. Use a level shifter if runs are long.

Do you want me to adjust pins and brightness for your wiring?

If you tell me which GPIOs your two NeoPixel strips are actually connected to on the XIAO and how you’re powering them, I’ll plug those in and test-compile settings for you. Also happy to add a debounce (e.g., require 2–3 consecutive sightings of a new area before switching) if you’ve got a noisy RF environment. [utsystemad...epoint.com]